National Cheng Kung University

Department of Electrical Engineering

Introduction to VLSI CAD (Spring 2023)

Lab Session 5

Synthesis of Sequential Logic and Some Tips

| Name | Student ID |  |  |
| --- | --- | --- | --- |
| 鄭智宇 | E94096110 |  |  |
| Practical Sections: |  | Points | Marks |
| ProbA |  | 30 |  |
| ProbB |  | 35 |  |
| ProbC |  | 35 |  |
| Notes |  |  |  |

Due Date: 14:59, March 22, 2023 @ moodle

Deliverables

- All Verilog codes including testbenches for each problem should be uploaded.

NOTE: Please DO NOT include source code in the paper report!

- All homework requirements should follow the naming rule in this file hierarchy or you will not get the full credit.

NOTE: Please DO NOT upload waveforms!

- Important! TA will use the command in Appendix A to check your design under SoC Lab environment, if your code can not be recompiled by TA successfully  using the commands, you will not get the full credit.

- If you upload a dead body, which we cannot even compile, you will get NO credit!

- All Verilog file before synthesizing should get at least 95% Superlint Coverage.

- Lab5_Student_ID.tar (English alphabet of Student_ID should be capital.)

Lab5_studentID.tar

Lab5_studentID

studentID.docx

ProbA

det_seq.v

det_seq_tb.v

det_seq_syn.v

det_seq_syn.sdf

ProbB

mini_vending.v

mini_vending_tb.v

mini_vending_syn.v

mini_vending_syn.sdf

ProbC

pattern_gen.v

pattern_gen_syn.v

pattern_gen_syn.sdf

pattern_gen_tb.v

Fig.1 File hierarchy for Homework submission

Objectives:

To make you be familiar with some designs of sequential logic and Design Complier. You can follow this document to practice. Please show your best.

Note that you can extend the spacing if it is not enough for you to answer.

All labs should be synthesized, and clock period should not over 10 ns.

ProbA: Design a circuit “detecting pattern 11001”

- Design a pattern seq-detecting circuit that can be synthesized with moore machine. The following is det_seq module’s specification. (Do NOT add or delete I/O ports, but you can change their behavior.)

- Please describe your FSM in detail

Explanation about your FSM

有限狀態機從初始狀態開始，根據輸入信號d進行狀態轉移，直到達到最終狀態。具體而言，有限狀態機的狀態轉移圖如下： s0狀態下，如果輸入信號d為1，則進入s1狀態，否則保持在s0狀態； s1狀態下，如果輸入信號d為1，則進入s2狀態，否則返回到s0狀態； s2狀態下，如果輸入信號d為1，則保持在s2狀態，否則進入s3狀態； s3狀態下，如果輸入信號d為1，則返回到s1狀態，否則進入s4狀態； s4狀態下，如果輸入信號d為1，則進入s5狀態，否則返回到s0狀態； s5狀態下，如果輸入信號d為1，則返回到s2狀態，否則保持在s0狀態。

- After synthesizing your design, you may have some information about the circuit. Please fill in the following form.

| Timing (slack) | Area (total cell area) | Power (total) |
| --- | --- | --- |
| 2.42 | 364.9 | 1.8079e-02mW |

- Please attach your design waveforms.

Your simulation result on the terminal.

Your waveform :

Explanation of your waveform :

q : 當有限狀態機進入s5狀態時，輸出信號q被置為1，表示達到了目標狀態。 num : 1. 用於計算有限狀態機達到最終狀態的次數。 2. 每當有限狀態機進入s5狀態時，計數器num會自增1。 3. 計數器的值被輸出到輸出端口num中。 4. 當復位信號rst被觸發時，計數器num被清零。

Superlint Coverage

Coverage : 100%

ProbB: Design a mini vending machine

- Design a mini vending machine with moore machine. The following is mini vending machine module’s specification. (Do NOT add or delete any I/O ports, but you can change their behavior.)

- Please describe your FSM in detail.

Explanation about your FSM

PHASE0：等待money輸入的狀態，若啟用輸入(en)，則轉移到PHASE1。 PHASE1：儲存beverage，直接轉移到PHASE2。 PHASE2：計算change，然後轉移到PHASE0。

- After synthesizing your design, you may have some information about the circuit. Please fill in the following form.

| Timing (slack) | Area (total cell area) | Power (total) |
| --- | --- | --- |
| 1.12 | 641.61 | 3.9283e-02mW |

- Please attach your design waveforms.

Your simulation result on the terminal.

Your waveform :

Explanation of your waveform :

PHASE0=2'd0; PHASE1=2'd1; PHASE2=2'd2; money_s 用來存儲投入的錢的暫存器，money_s 用於計算剩餘的錢。 1.當狀態機處於 PHASE0 時， money_s 就會被設置為 money。 2.當狀態機處於 PHASE1 時， money_s 就會減去 beverage 的值。 change : 用於存儲需要找給使用者的錢。 1.當狀態機處於 PHASE2 時， change 就會被設置為 money_s。 2.在其它狀態下， change就會被設置為 0。 Finish : 用於表示交易是否完成。 1.當狀態機處於 PHASE2 時， finish 就會被設置為 1，表示交易完成。 2.在其它狀態下， finish 就會被設置為 0。

Superlint Coverage

1 warning Coverage : 98.5%

ProbC: Design a pattern generator

- Design a pattern generator which can create pattern 1100, 1101, 1110, 1111 and use mealy machine. The following is pattern generator specification.

- Please describe your FSM in detail.

Explanation about your FSM

1. 分成 5 個狀態 2. 初始狀態為 INIT，在該狀態下如果 en 信號為 1，則轉移到 s0 狀態；否 則保持在 INIT 狀態。 3. 其餘狀態的轉移均為循環狀態，即 s0 -> s1 -> s2 -> s3 -> INIT –(en)> s0…。

- After synthesizing your design, you may have some information about the circuit. Please fill in the following form.

| Timing (slack) | Area (total cell area) | Power (total) |
| --- | --- | --- |
| 3.61 | 162.95 | 1.1065e-02mW |

- Please attach your design waveforms.

Your simulation result on the terminal.

Your waveform :

Explanation of your waveform :

除了初始狀態 INIT 和循環狀態 s0 外，每個狀態都有不同的輸出信號。 1.pattern 當狀態為 s2 時，輸出信號 pattern 為 sel 的第 2 位； 當狀態為 s3 時，輸出信號 pattern 為 sel 的第 1 位； 在其餘狀態下，pattern 輸出為 1。 2.valid 除了 INIT 狀態外，信號始終與 en 信號同步，即當 en 為 1 時，valid 為 1。

Superlint Coverage

Coverage : 100%

- At last, please write the lessons learned from this lab session, or some suggestions for this lab session. Thank you.

學習到verdi的使用方法與狀態機的應用

Appendix A : Commands we will use to check your homework